[image: image3.png]VAMIAXIS



                                       FPR-861 Product Specification

FAP60 Fingerprint Scanner 

FPR-861
Product Specification
Version：V1.2
[image: image1.jpg]



MIAXIS BIOMETRICS CO.,LTD
Add:5 Floor, Science&technology Building, East Software Park, No.90, Wensan Rd, Hangzhou, 310012, China Tel: +86-571-81951610       Email: inquiry@miaxis.com      http://www.miaxis.net
	Document Revision History

	Version
	Date
	Comment

	V1.0
	7/2023
	First issue

	V1.1
	11/2023
	Update Technical Parameters

	V1.2
	12/2023
	Update Technical Parameters and Mechanical Dimension


Content
41.
Product Overview


41.1
Brief Introduction


41.2
Application


42.
Product Specification


42.1
Technical Parameters


52.2
Mechanical Dimension





1. Product Overview
1.1 Brief Introduction
MIAXIS® FPR-861 is the FBI Appendix F certified, compliant with the MOSIP Registered Device Specification, Ultra slim, and Lightweight FAP60 Fingerprint Scanner, born for ID authentication system. Combined with MIAXIS® fingerprint algorithm and an advanced TFT technology. MIAXIS® FPR-861 supports large area fingerprint capture, enables capture of super high fingerprint image quality. Ideal for biometric authentication and national identity verification programs.
1.2 Application
Widely used for information security, e-ID, identity verification industries.
2. Product Specification
1.3  Technical Parameters
	Specification
	Detail

	Dimension (W x L x H)
	138.0mm x 129.0mm x 20.5mm

	Capture Size (W x L)
	81.28mm x 76.2mm(4-Finger)

	Image Size (W x H)
	4-Finger: 1600 x 1500
Roll: 800 x 750

	FPS
	≥2(4-Finger)    ≥10(Rolls)

	Image Quality Standards
	FBI IQS Appendix F

	Fingerprint Types
	Four Finger Flat / Two Finger Flat / Single Flat & Rolls

	Image Resolution
	500 dpi

	Grey Scale
	8bit, 256 levels

	Image Format
	WSQ, JPEG2000, RAW, BMP, ISO 19794-4, ANSI 381

	Compression Ratio
	Up to 15:1

	Quality Score
	NFIQ v1.0

	Ingress Protection
	IP65（Sensor Surface)

	Surface Hardness
	9H

	Operating Temperature
	-10℃ to +50℃

	Storage Temperature
	-20℃ to +60℃

	Operating Humidity
	20％RH～90％RH, non-condensing

	ESD Immunity
	±8kV contact discharge, ±15kV air discharge

	Interface
	USB2.0 Type-A

	Accessories
	Power indication*1, Status indicator*4, Buzzer

	Power Supply
	DC5V±5% (USB powered)

	Supply Current
	≤600mA (USB powered)

	Operating System Environment
	Android 6 or higher
Windows 10 or higher 

Linux


1.4  Mechanical Dimension
[image: image2.png]



Mechanical Dimension（Unit: mm）         
Note：The device FPR-861 supports standard USB communication interface, with 1.5m length USB cable (cable length optional)

 2 / 2

[image: image3.png]